■Product Information

Product Name：19 Inch ODF

Product Model：24C-SC-1U- Type

■Applications

▶Optical Access Network

▶Data Processing Centers

▶Cable television

▶Local Area Network

▶Coarse Wavelength Division Multiplexer

■Features

▶Design of fixed panel

▶Drawing type structure, convenient construction operation

▶Fit for“19”rack.

▶Suitable for FC,SC,LC,ST,etc

■Configuration

| Item |  | Specification | Remark |
| --- | --- | --- | --- |
| Size（mm） |  | 435*315*1U |  |
| Fiber core number | LC Pigtail | 48(DX) |  |
|  | SC Pigtail | 24（SX） |  |
| Cassette |  | 1*24C or 2*12C or 2*24C |  |
| Material |  | Cold-Rolled steel plate |  |
| Thickness of material（mm） |  | 1.2 |  |
| Weight（KG） |  | 3.6 |  |
| Color |  | Black |  |
| Color Number |  | / |  |
| Operating Temperature（℃） |  | -40～+65 |  |

■Optical Parameter

| Item |  | Characteristics | Remarks |
| --- | --- | --- | --- |
| Pigtail | Fiber Type | G.657A1 |  |
|  | Length of | 1.5M |  |
|  | Color | multicolour |  |
| Insertion Loss |  | ≤0.3dB |  |
| Return Loss |  | SM:APC≥60dB,UPC≥50dB |  |
| Repeatability |  | ≤0.2dB |  |
| Interchange |  | ≤0.2dB |  |
| Operation Temperature |  | -30～75℃ |  |
| Relative Humidity |  | ≤85% |  |

■Cable Route

The branch cable is fused through the

molten disc in the box and the installed

cable tail fiber.

.

■Accessories

- cassette：1*24C or 2*12C or 2*24C

- Clamps：4 Set.

- fixing bolts: 4 pcs

- SC adapters: 24 pcs SC simplex

- heat shrink: 24pcs